Regulations Governing Satellite Accumulation Areas

RCRA (40 CFR 262)

| Waste is accumulated at or near the point of generation under the control of the operator. | 262.15(a) |
| --- | --- |
| Container(s) have less than 55 gallons of Hazardous Waste. | 262.15(a) |
| Container(s) have less than 1quart or 1kilogram of acutely hazardous waste. | 262.15(a) |
| Container(s) are in good condition and non-leaking. | 262.15(a)(1) |
| Waste is compatible with the container it is stored in. | 262.15(a)(2) |
| Only compatible wastes are stored in each container and the container is separated from nearby incompatible materials. | 262.15(a)(3) |
| Container is closed except when adding, removing, or consolidating waste or when temporary venting is necessary. | 262.15(a)(4) |
| Container is marked with the words “Hazardous Waste” and an indication of the hazards of the contents. | 262.15(a)(5) |
| Satellite area complies with all preparedness and prevention requirements noted below. | 262.15(a)(8) |
| Area is maintained to prevent fire, explosions, and spills. | 262.16(b)(8)(i) |
| Area must be equipped with: - Internal communications to signal to facility personnel. - Communication device to alert local emergency response personnel. - Fire extinguishers, spill control equipment, and decontamination equipment. - Fire suppression, adequate water supply or foam producing equipment. | 262.16(b)(8)(ii) |
| Regular testing and maintenance of equipment is performed. | 262.16(b)(8)(iii) |
| Immediate access to communication equipment when handling hazardous waste. | 262.16(b)(8)(iv) |
| Adequate aisle space. | 262.16(b)(8)(v) |
| Emergency information posted next to telephones or in areas directly involved in the generation and accumulation of hazardous waste. | 262.16(b)(9)(ii) |